CXIS

Complex eXtensive Instruction Set

Architecture Specification  —  v1.0 Draft

Quick Reference Card

| property | value |
| --- | --- |
| word size | 32-bit (primary), 64-bit (long ops) |
| operand order | instr src1, src2, dst (destination last) |
| mnemonic case | lowercase always |
| register file | 206 registers across 7 classes |
| instruction encoding | variable-length (1 opcode + 1 mod byte + operands) |
| memory model | von neumann — shared instruction + data |
| endianness | little-endian |
| comment syntax | ; single line comment |
| label syntax | name: (colon-terminated) |
| section syntax | section name: |

# 1. register file

CXIS has 206 registers organized into 7 classes. register names are always lowercase. each class serves a specific purpose and type-safety is enforced at the instruction level.

## 1.1 general-purpose registers

| class | range | width | type | count | purpose |
| --- | --- | --- | --- | --- | --- |
| integer 32-bit | i0 – i31 | 32-bit | signed / unsigned | 32 | general arithmetic, addressing |
| integer 64-bit | l0 – l31 | 64-bit | signed / unsigned | 32 | wide ops, pointers, long math |
| float 32-bit | f0 – f31 | 32-bit | ieee 754 single | 32 | single-precision floating point |
| double 64-bit | d0 – d31 | 64-bit | ieee 754 double | 32 | double-precision floating point |
| condition | c0 – c31 | 32-bit | boolean / flags | 32 | branch predicates, compare results |
| scratch | s0 – s31 | 32-bit | any / volatile | 32 | caller-saved temporaries |
| argument | a0 – a9 | 32/64 | caller / callee | 10 | function args and return values |

## 1.2 protected special-purpose registers

these registers are protected and cannot be freely assigned by user code. they are managed by the ABI and operating system.

| register | full name | width | description |
| --- | --- | --- | --- |
| sp | stack pointer | 32-bit | points to top of current stack frame |
| sf | stack frame | 32-bit | base of current stack frame (callee-saved) |
| bp | base pointer | 32-bit | base of data segment |
| bf | base frame | 32-bit | base frame for nested call tracking |

## 1.3 calling convention

argument registers follow this convention:

| register | role |
| --- | --- |
| a0 | 1st argument / return value |
| a1 | 2nd argument / 2nd return value (for wide returns) |
| a2 – a7 | 3rd through 8th arguments |
| a8 – a9 | additional args (caller-saved, spilled if needed) |
| s0 – s31 | scratch — caller-saved, freely clobbered across calls |
| i0 – i15 | callee-saved — must be preserved across calls |
| i16 – i31 | caller-saved — may be clobbered |

# 2. instruction encoding

CXIS uses variable-length encoding. every instruction starts with a fixed 2-byte header (opcode + mod byte), followed by a variable number of operand bytes.

## 2.1 instruction format

| byte(s) | field | size | description |
| --- | --- | --- | --- |
| 0 | opcode | 1 byte | instruction opcode (0x00–0xff) |
| 1 | mod byte | 1 byte | operand count, types, and addressing mode |
| 2+ | operands | variable | register ids, immediates, or memory refs |

## 2.2 mod byte layout

the mod byte encodes operand count and type information in 8 bits:

| bits | field | values | description |
| --- | --- | --- | --- |
| [7:6] | operand count | 00=0 01=1 10=2 11=3 | number of explicit operands |
| [5:4] | dst type | 00=i 01=l 10=f 11=d | destination register class |
| [3:2] | src1 type | 00=i 01=l 10=f 11=d | first source register class |
| [1:0] | src2 type | 00=i 01=l 10=f 11=d | second source register class |

## 2.3 operand encoding

| operand kind | size | encoding |
| --- | --- | --- |
| register | 1 byte | register id 0–205 (maps to all 206 registers) |
| immediate i32 | 4 bytes | little-endian signed 32-bit integer |
| immediate i64 | 8 bytes | little-endian signed 64-bit integer |
| immediate f32 | 4 bytes | ieee 754 single-precision float |
| immediate f64 | 8 bytes | ieee 754 double-precision float |
| memory ref | 5 bytes | 1 byte mode + 4 byte offset (base reg in mod byte) |
| label / offset | 4 bytes | signed 32-bit relative offset from next instr |

## 2.4 instruction size examples

| instruction | bytes | breakdown |
| --- | --- | --- |
| nop | 1 | opcode only |
| ret | 1 | opcode only |
| inc i0 | 3 | opcode + mod + reg(1) |
| mov i0, i1 | 4 | opcode + mod + reg(1) + reg(1) |
| add i0, i1, i2 | 5 | opcode + mod + reg(1) + reg(1) + reg(1) |
| mov 0xff, i0 | 7 | opcode + mod + imm32(4) + reg(1) |
| mov 0xff, l0 | 11 | opcode + mod + imm64(8) + reg(1) |

# 3. operand order

CXIS uses AT&T / src-first style. the destination register is always last. sources read left to right, result lands on the right — like math: x + y → z

## 3.1 general rule

instr src1, src2, dst        ; 3-operand: dst = src1 op src2

instr src, dst               ; 2-operand: dst = op(src)

instr dst                    ; 1-operand: dst = op(dst)  e.g. inc

instr                        ; 0-operand: e.g. nop, ret, halt

## 3.2 examples

add   i0, i1, i2             ; i2 = i0 + i1

fadd  f0, f1, f2             ; f2 = f0 + f1

dadd  d0, d1, d2             ; d2 = d0 + d1

mov   i0, i1                 ; i1 = i0

mov   0xff, i0               ; i0 = 0xff  (immediate)

mov   0x3f800000, f0         ; f0 = 1.0   (float immediate)

lea   [bp+8], i0             ; i0 = address of [bp+8]

not   i0, i1                 ; i1 = ~i0

shl   i0, 2, i1              ; i1 = i0 << 2

cmp   i0, i1, c0             ; c0 = compare(i0, i1)

je    c0, my_label           ; if c0 == 1, jump

push  i0                     ; push i0 (stack is implicit dst)

pop   i1                     ; pop  i1 (stack is implicit src)

inc   i0                     ; i0 = i0 + 1  (src == dst)

call  my_func                ; push ret addr, jump

# 4. sections and labels

## 4.1 standard sections

| section | purpose | typical contents |
| --- | --- | --- |
| section text: | executable code | instructions, function bodies |
| section data: | initialized data | variables, strings with known values |
| section rodata: | read-only constants | string literals, float constants, lookup tables |
| section bss: | uninitialized / zero | buffers, large arrays — no initial value stored |

## 4.2 label types

| label kind | syntax | scope | description |
| --- | --- | --- | --- |
| global label | global name: | file-wide / exported | entry points, exported functions |
| local label | name: | file-local | loop targets, branch destinations |

## 4.3 data definition directives

| directive | size | section | description |
| --- | --- | --- | --- |
| db val | 1 byte | data / rodata | define byte |
| dw val | 2 bytes | data / rodata | define word (16-bit) |
| dd val | 4 bytes | data / rodata | define dword (32-bit) |
| dq val | 8 bytes | data / rodata | define qword (64-bit) |
| df val | 4 bytes | rodata | define float (ieee 754 single) |
| resb n | n bytes | bss | reserve n bytes (zero-initialized) |
| resw n | 2n bytes | bss | reserve n words |
| resd n | 4n bytes | bss | reserve n dwords |
| resq n | 8n bytes | bss | reserve n qwords |

## 4.4 full program example

section rodata:

global msg:

db 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x00   ; "Hello\0"

section data:

global counter:

dd 0x00000000                             ; int counter = 0

section bss:

global buffer:

resb 256                                  ; byte buffer[256]

section text:

global _start:

mov  0x0, i0                              ; i0 = 0

mov  0xa, i1                              ; i1 = 10  (loop limit)

loop_top:

cmp  i0, i1, c0                           ; c0 = (i0 < i1)

jge  c0, loop_end                         ; if i0 >= i1, exit

inc  i0                                   ; i0++

goto loop_top

loop_end:

mov  i0, a0                               ; return value = i0

exit a0                                   ; exit(i0)

# 5. instruction set summary

## 5.1 data movement  (0x01 – 0x0d)

| mnemonic | opcode | operands | description |
| --- | --- | --- | --- |
| mov | 0x01 | src, dst | dst = src |
| movsx | 0x02 | src, dst | dst = sign_extend(src) |
| movzx | 0x03 | src, dst | dst = zero_extend(src) |
| movsxd | 0x04 | src, dst | dst = sign_extend_32_to_64(src) |
| push | 0x05 | src | stack[--sp] = src |
| pop | 0x06 | dst | dst = stack[sp++] |
| pusha | 0x07 | — | push all i-registers |
| popa | 0x08 | — | pop all i-registers |
| lea | 0x09 | [mem], dst | dst = effective_address(mem) |
| fmov | 0x0a | src, dst | dst = src (float) |
| dmov | 0x0b | src, dst | dst = src (double) |
| lmov | 0x0c | src, dst | dst = src (int64) |
| cmov | 0x0d | cx, src, dst | if cx: dst = src |

## 5.2 arithmetic  (0x10 – 0x28)

| mnemonic | opcode | operands | description |
| --- | --- | --- | --- |
| add | 0x10 | src1, src2, dst | dst = src1 + src2 (i32) |
| addc | 0x11 | src1, src2, dst | dst = src1 + src2 + carry |
| sub | 0x12 | src1, src2, dst | dst = src1 - src2 (i32) |
| subb | 0x13 | src1, src2, dst | dst = src1 - src2 - borrow |
| mul | 0x14 | src1, src2, dst | dst = src1 * src2 (unsigned) |
| imul | 0x15 | src1, src2, dst | dst = src1 * src2 (signed) |
| div | 0x16 | src1, src2, dst | dst = src1 / src2 (unsigned) |
| idiv | 0x17 | src1, src2, dst | dst = src1 / src2 (signed) |
| inc | 0x18 | dst | dst = dst + 1 |
| dec | 0x19 | dst | dst = dst - 1 |
| neg | 0x1a | src, dst | dst = -src |
| ladd | 0x1b | src1, src2, dst | dst = src1 + src2 (i64) |
| lsub | 0x1c | src1, src2, dst | dst = src1 - src2 (i64) |
| lmul | 0x1d | src1, src2, dst | dst = src1 * src2 (i64) |
| ldiv | 0x1e | src1, src2, dst | dst = src1 / src2 (i64) |
| fadd | 0x1f | src1, src2, dst | dst = src1 + src2 (f32) |
| fsub | 0x20 | src1, src2, dst | dst = src1 - src2 (f32) |
| fmul | 0x21 | src1, src2, dst | dst = src1 * src2 (f32) |
| fdiv | 0x22 | src1, src2, dst | dst = src1 / src2 (f32) |
| fneg | 0x23 | src, dst | dst = -src (f32) |
| dadd | 0x24 | src1, src2, dst | dst = src1 + src2 (f64) |
| dsub | 0x25 | src1, src2, dst | dst = src1 - src2 (f64) |
| dmul | 0x26 | src1, src2, dst | dst = src1 * src2 (f64) |
| ddiv | 0x27 | src1, src2, dst | dst = src1 / src2 (f64) |
| dneg | 0x28 | src, dst | dst = -src (f64) |

## 5.3 bitwise / shift  (0x30 – 0x46)

| mnemonic | opcode | operands | description |
| --- | --- | --- | --- |
| and | 0x30 | src1, src2, dst | dst = src1 & src2 |
| or | 0x31 | src1, src2, dst | dst = src1 ¦ src2 |
| xor | 0x32 | src1, src2, dst | dst = src1 ^ src2 |
| not | 0x33 | src, dst | dst = ~src |
| shl | 0x34 | src, n, dst | dst = src << n |
| shr | 0x35 | src, n, dst | dst = src >> n (logical) |
| sar | 0x36 | src, n, dst | dst = src >> n (arithmetic) |
| rol | 0x37 | src, n, dst | dst = rotate_left(src, n) |
| ror | 0x38 | src, n, dst | dst = rotate_right(src, n) |
| bsf | 0x3d | src, dst | dst = lowest set bit index |
| bsr | 0x3e | src, dst | dst = highest set bit index |
| popcnt | 0x43 | src, dst | dst = popcount(src) |
| lzcnt | 0x44 | src, dst | dst = leading zeros |
| tzcnt | 0x45 | src, dst | dst = trailing zeros |
| test | 0x46 | src1, src2, cx | flags only — no write |

## 5.4 compare & set  (0x50 – 0x5c)

| mnemonic | opcode | operands | description |
| --- | --- | --- | --- |
| cmp | 0x50 | src1, src2, cx | integer compare → condition reg |
| lcmp | 0x51 | src1, src2, cx | i64 compare → condition reg |
| fcmp | 0x52 | src1, src2, cx | float compare → condition reg |
| dcmp | 0x53 | src1, src2, cx | double compare → condition reg |
| eq | 0x54 | src1, src2, cx | cx = 1 if src1 == src2 |
| ne | 0x55 | src1, src2, cx | cx = 1 if src1 != src2 |
| gt | 0x56 | src1, src2, cx | cx = 1 if src1 > src2 |
| lt | 0x57 | src1, src2, cx | cx = 1 if src1 < src2 |
| gte | 0x58 | src1, src2, cx | cx = 1 if src1 >= src2 |
| lte | 0x59 | src1, src2, cx | cx = 1 if src1 <= src2 |

## 5.5 control flow  (0x60 – 0x70)

| mnemonic | opcode | operands | description |
| --- | --- | --- | --- |
| jmp | 0x60 | label | unconditional jump |
| goto | 0x61 | label | alias for jmp |
| jcc | 0x62 | cx, label | jump if cx is true |
| je | 0x63 | cx, label | jump if equal |
| jne | 0x64 | cx, label | jump if not equal |
| jg | 0x65 | cx, label | jump if greater |
| jge | 0x66 | cx, label | jump if greater or equal |
| jl | 0x67 | cx, label | jump if less |
| jle | 0x68 | cx, label | jump if less or equal |
| ja | 0x69 | cx, label | jump if above (unsigned) |
| jb | 0x6a | cx, label | jump if below (unsigned) |
| loop | 0x6b | cx, label | dec i0, jump if i0 != 0 |
| call | 0x6c | label | push ret addr, jump |
| ret | 0x6d | — | return from subroutine |
| retn | 0x6e | n | return, pop n bytes |
| exit | 0x6f | src | exit with code in src |
| halt | 0x70 | — | halt cpu until interrupt |

## 5.6 misc / system  (0x80 – 0x8d)

| mnemonic | opcode | operands | description |
| --- | --- | --- | --- |
| nop | 0x80 | — | no operation |
| wait | 0x81 | — | wait for pending ops |
| in | 0x82 | port, dst | read i/o port → dst |
| out | 0x83 | src, port | write src to i/o port |
| int | 0x84 | n | software interrupt |
| iret | 0x85 | — | return from interrupt |
| cpuid | 0x86 | — | cpu info → a0–a3 |
| rdtsc | 0x87 | — | timestamp counter → l0 |
| pause | 0x88 | — | spin-wait hint |
| ud | 0x89 | — | undefined — guaranteed fault |
| align | 0x8a | n | align to n-byte boundary |
| section | 0x8b | name | switch assembler section |
| global | 0x8c | sym | export symbol |
| local | 0x8d | sym | file-local symbol |

# 6. memory addressing modes

| mode | syntax | description |
| --- | --- | --- |
| immediate | 0xff | constant value literal |
| register | i0 | value in register |
| direct | [0x1000] | value at absolute address |
| register indirect | [i0] | value at address in register |
| base + offset | [i0+8] | address = reg + constant offset |
| base + index | [i0+i1] | address = base reg + index reg |
| scaled index | [i0+i1*4] | address = base + (index * scale) |
| bp-relative | [bp+8] | argument access via base pointer |
| sf-relative | [sf-4] | local variable via stack frame |

; examples

mov  [i0], i1               ; i1 = memory[i0]

mov  i1, [i0]               ; memory[i0] = i1

mov  [bp+8], i0             ; i0 = first argument

mov  i2, [sf-4]             ; store to local variable

add  [i0+i1*4], i2, i3     ; i3 = mem[i0 + i1*4] + i2

# 7. opcode map

full opcode allocation across all ranges:

| range | group | count |
| --- | --- | --- |
| 0x00 | reserved / nop | 1 |
| 0x01 – 0x0d | data movement | 13 |
| 0x0e – 0x0f | reserved (future) | 2 |
| 0x10 – 0x28 | arithmetic (i32/i64/f/d) | 25 |
| 0x29 – 0x2f | reserved (future) | 7 |
| 0x30 – 0x46 | bitwise and shift | 23 |
| 0x47 – 0x4f | reserved (future) | 9 |
| 0x50 – 0x5c | compare and set | 13 |
| 0x5d – 0x5f | reserved (future) | 3 |
| 0x60 – 0x70 | control flow | 17 |
| 0x71 – 0x77 | reserved (future) | 7 |
| 0x78 – 0x7f | string operations | 8 |
| 0x80 – 0x8d | misc and system | 14 |
| 0x8e – 0xef | reserved for extensions | 98 |
| 0xf0 – 0xff | reserved for SIMD future | 16 |

end of specification  —  CXIS v1.0 draft